# L1-Overview

### Level 1 – Jetson Foundations

Workshop Overview

Duration: 5 full days (typically 9:00 AM – 5:00 PM, with breaks and lunch)

Target age: 15+ (high-school students, gap-year learners, early university students)

Group size: 12–24 participants (ideally 3–4 per team)

Core philosophy: 80–90% hands-on. Students physically build, wire, debug, and program real robots using the same NVIDIA Jetson hardware found in industry and research labs. No toy platforms — everything is commercial-grade.

Robots used (all three introduced from Day 1):

- Rover (wheeled differential-drive platform) – main focus Days 1 & 3–4

- Dog (12-DOF quadruped) – main project Day 2 + used in missions Days 3–5

- Humanoid (bipedal servo-based) – live demonstrations every day (balance, walking, gestures); students observe and get inspired, but do not fully build/program it in Level 1

Key skills acquired:

- Safe assembly and wiring of embedded robotics platforms

- Flashing and configuring NVIDIA JetPack on Jetson Orin Nano

- Python programming for real-time robot control (GPIO, PWM, sensor reading)

- Basic closed-loop control (PID) for speed, heading, and balance

- Simple color-based vision using pure Python (Pillow)

- Finite state machines for behavior control

- Basic multi-robot coordination and communication

- Debugging real hardware issues under time pressure

- Teamwork, presentation, and ethical reflection

Culminating project:

“Dog-Led Urban Search & Rescue”

A fully autonomous quadruped robot navigates an arena, detects an object via simple vision or ultrasonic, and completes a mock rescue — all designed, built, and coded by the students.

Outcomes for students:

- Tangible portfolio: 3–5 mission videos + Python code repositories

- Confidence from debugging real hardware failures

- Strong foundation for Level 2 (autonomy & ROS 2)

- Competitive edge for university applications, robotics clubs, and internships

Unique selling points:

- Real NVIDIA Jetson hardware from Day 1 (not Arduino or Raspberry Pi Pico)

- Multi-locomotion progression (wheeled → quadruped → humanoid exposure)

- No OpenCV in Level 1 — keeps vision simple and accessible

- 3 months post-workshop Webots simulation access for continued learning

### Hardware List – Level 1

Per team of 3–4 students (recommended ratio 1 kit : 3–4 participants)

- NVIDIA Jetson Orin Nano Developer Kit (8 GB or Super variant) – 1 per team

- Jetson-compatible power supply (5V 4A+ barrel jack or USB-C PD) – 1

- NVMe SSD (128 GB–512 GB, M.2 2230/2242) + adapter if needed – 1

- Wheeled Rover chassis kit (JetBot-style or Hiwonder JetRover equivalent)

  - Includes: chassis plates, 4× DC motors + encoders, wheels, motor driver (e.g., L298N or TB6612)

- Quadruped Dog chassis kit (Petoi Bittle-style or similar 12-DOF open-source quadruped)

  - 12× high-voltage metal-gear servos (e.g., STS3215 or equivalent)

  - Servo extension cables

  - PCA9685 16-channel PWM driver board (I2C)

- Bipedal Humanoid demo unit (shared class-wide, 1–2 units total)

  - 16–20 DOF servo-based biped (pre-built or semi-assembled for demo only)

- Sensors (per team):

  - 2–3× HC-SR04 ultrasonic sensors

  - 1× MPU6050 or BNO055 IMU (I2C)

  - 2× IR distance/reflectance sensors (TCRT5000 or similar)

  - 2–4× mechanical limit switches (microswitches)

- CSI camera module (Raspberry Pi Camera v2 or Arducam IMX219 compatible with Jetson) – 1–2 per team

- Power:

  - 7.4V–11.1V LiPo battery pack (2200–5000 mAh) + balance charger

  - 5V UBEC/step-down for Jetson logic power

- Wiring & tools (per team + shared):

  - Jumper wires (male-female, male-male, female-female)

  - Breadboard or protoboard

  - Screwdriver set (Phillips #0–2, hex keys)

  - Electrical tape, zip ties, hot glue gun + sticks

Shared class-wide (for 12–24 students):

- 2–3× spare Jetson Orin Nano units (for emergencies / flashing station)

- 1–2× HDMI monitors + keyboards/mice (or use student laptops via SSH/VNC)

- Extra batteries, servos, cables, fuses

- Arena materials: cardboard boxes, ramps, colored cards/balls, tape lines

### Software / Program List – Level 1

Installed on each Jetson (via JetPack 6.x image):

- Ubuntu 22.04 LTS (base OS from JetPack)

- NVIDIA JetPack 6.0 / 6.1 / 6.2 (includes CUDA, cuDNN, TensorRT, etc.)

- Python 3.10 (default)

- pip (Python package manager)

Required Python libraries (install via pip on Jetson):

Bash

pip install jetbot pillow numpy adafruit-circuitpython-servokit jetson-utils

- jetbot (optional helper library for Rover, or use raw GPIO)

- pillow (PIL fork – for simple image processing / color check)

- numpy (array operations for image data)

- adafruit-circuitpython-servokit (PCA9685 PWM control for Dog servos)

- jetson-utils (NVIDIA utilities for CSI camera access)

Development tools (pre-installed or easily added):

- Visual Studio Code (via apt or .deb from NVIDIA)

- JupyterLab / Jupyter Notebook (optional for quick testing)

- nano / vim (terminal editors)

- git (for version control – optional but recommended)

Simulation software (post-workshop home access):

- Webots (free open-source robot simulator) – students get 3-month guided access with custom ChatoorAI worlds matching their robots

No additional heavy dependencies in Level 1:

- No OpenCV (removed as requested)

- No ROS 2 (saved for Level 2)

### Miscellaneous Items Required for Workshop

Facility / Venue Needs

- Classroom/lab with tables (1 table per 3–4 students)

- Power strips / extension cords (multiple outlets per table)

- Good Wi-Fi (for SSH and occasional package installs)

- Projector or large screen for live demos

- Whiteboard + markers

- First-aid kit (minor cuts from screws possible)

Consumables (estimate for 20 students)

- Electrical tape rolls (4–6)

- Zip ties (200–300)

- Hot glue sticks (50–100)

- Spare jumper wires (500+ assorted)

- Batteries (LiPo 3S 2200–5000 mAh × 6–8 spares)

- MicroSD cards (if not using NVMe) – 64 GB+ × 2–3 spares

Safety & Comfort

- Safety glasses (recommended for servo/gear work)

- Anti-static wrist straps (optional but good practice)

- Water bottles & snacks area (away from hardware)

- Fan or air conditioning (Jetson + servos can get warm)

Documentation & Admin

- Printed daily handouts/workbooks (1 set per student)

- Level 1 certificates (pre-printed or digital)

- Attendance sheet / waiver forms

- Photo release forms (if posting student work online)

# Day 1

# ChatoorAI Academy –

## Level 1: Jetson Foundations

Goal of Day 1: Create instant excitement, build confidence, and give every student a tangible “I just made a real robot move!” moment within the first 4 hours.

### Day 1 Learning Objectives

By the end of Day 1, students will be able to:

- Identify the main components of a commercial robotics platform (NVIDIA Jetson Orin Nano, motors, sensors, chassis).

- Safely assemble and wire a basic wheeled Rover robot.

- Flash JetPack 6.x and run their first Python script on real Jetson hardware.

- Control a robot using simple timed motor commands.

- Understand the difference between simulation and real hardware.

- Feel excited and confident about the rest of the workshop.

### Materials Needed (per team of 4 students)

- 1× NVIDIA Jetson Orin Nano Super Developer Kit (pre-flashed if possible, or one host PC for flashing)

- 1× Rover chassis kit (JetBot-style or Hiwonder JetRover equivalent)

- Motors, wheels, battery pack, power distribution board

- Ultrasonic sensor + IMU (pre-soldered or with clear pin labels)

- Screwdrivers, pliers, zip ties, electrical tape

- HDMI monitor + keyboard/mouse (or SSH setup)

- USB-C power supply for Jetson

- Team name cards, stickers, small prizes for “best victory dance”

- Projector/screen for demos

Teacher Prep (do this the day before):

- Pre-flash at least one Jetson as demo unit

- Test one complete Rover build

- Prepare a 3-minute hype video (quick clips of robots moving + upbeat music)

- Print simple wiring diagrams (color-coded)

### Detailed Day 1 Schedule (9:00 AM – 5:00 PM)

9:00 – 9:30 AM | Welcome & High-Energy Kickoff (30 min)

- Greet students at the door with music playing and robots already on display (Rover doing circles, Dog walking, Humanoid waving).

- Quick icebreaker: “Stand up, say your name and which robot you’re most excited to build today.”

- Show 2-minute hype video.

- Explain the big picture: “Over 5 days you will build 3 real robots using the same hardware used by Tesla and Boston Dynamics.”

9:30 – 10:15 AM | Robot Reveal & All-Three Introduction (45 min)

Teacher Script (very important to deliver with energy):

“Today we’re not just learning theory — we’re becoming robot builders.

We have three robots:

- Rover – the explorer (your main build today)

- Dog – the walker (your main project tomorrow)

- Humanoid – the most advanced (you’ll see it do amazing things every day)”

Live demos (5 min each):

- Rover does obstacle course

- Dog walks and turns

- Humanoid waves, balances, and does a simple dance

10:15 – 10:45 AM | Jetson Orin Nano Basics & Safety Talk (30 min)

- What is Jetson? (Edge AI computer – brain of modern robots)

- Safety rules: Never touch moving parts while powered, careful with batteries, no food/drink near hardware.

- Quick tour of Jetson ports and power button.

10:45 – 11:00 AM | Break

11:00 AM – 1:00 PM | Rover Assembly & First Code (2 hours – most important hands-on block)

Step-by-step guided build (teacher demos on projector, students follow):

- Chassis Assembly (20 min)

  - Mount motors and wheels

  - Attach battery holder and power board

- Jetson Mounting & Wiring (30 min)

  - Secure Jetson board

  - Connect motor driver to Jetson GPIO

  - Wire ultrasonic sensor and IMU

- First Boot & JetPack Check (15 min)

  - Power on → login → run jtop to show performance

First Python Script (30 min)
Python
from jetbot import Robot

robot = Robot()

robot.forward(0.3)   # 30% speed

time.sleep(2)

- robot.stop()
Students modify speed and duration.

- Victory Dance Challenge (15 min) Students code their Rover to spin, go forward, stop, and blink lights if available. Best dance wins a small prize + loud applause.

1:00 – 2:00 PM | Lunch

2:00 – 3:30 PM | Rover Mission: “Rover Recon” (90 min)

- Set up a simple arena with 3 obstacles and a target zone.

- Students improve their code to complete the mission cleanly.

- Teacher circulates, helps with debugging.

- Record every team’s successful run on phone/video.

3:30 – 4:00 PM | Reflection & Humanoid Teaser (30 min)

- Group debrief: “What was hardest? What felt amazing?”

- Final 5-minute Humanoid demo (advanced movements to build anticipation for later days).

4:00 – 5:00 PM | Wrap-up, Photos & Homework

- Group photo with all robots

- Homework: Watch the 3-minute recap video (you record today’s highlights) and think of a team name for tomorrow’s Dog build.

- Hand out Level 1 Day 1 sticker/certificates.

# Day 2

## Day 2 Learning Objectives

By the end of Day 2, students will be able to:

- Fully assemble a 12-DOF quadruped robot (Dog) mechanically and electrically.

- Safely connect and calibrate high-voltage servos to the NVIDIA Jetson Orin Nano.

- Understand basic inverse kinematics concepts through practical gait coding.

- Write and debug Python code to make the Dog walk and trot on real hardware.

- Coordinate basic multi-robot behavior (Dog following Rover).

- Appreciate the difference between wheeled and legged locomotion.

### Materials Needed (per team of 4 students)

- 1× Pre-assembled or partially assembled quadruped chassis (Petoi Bittle-style or similar 12-DOF kit)

- 12× High-voltage metal-gear servos (pre-installed or ready to mount)

- Servo extension cables, PCA9685 PWM driver board (if not using Jetson direct PWM)

- Battery pack (LiPo or 7.4V–11.1V suitable for servos)

- Screwdrivers (Phillips #1 & #2), hex keys, pliers, zip ties

- Jetson Orin Nano (from Day 1 Rover, or shared team unit)

- Monitor/keyboard or SSH laptop for coding

- Ruler/tape measure (for calibration)

- Small obstacles (books, boxes) for testing stability

- Team name stickers from Day 1 + “Dog Builder” badges/stickers for completion

Teacher Prep (do this the night before):

- Test one fully working Dog (walk/trot cycles stable).

- Pre-label servo cables if not color-coded.

- Prepare 2-minute “Dog walking compilation” video for inspiration.

- Have spare servos and cables ready for quick swaps.

### Detailed Day 2 Schedule (9:00 AM – 5:00 PM)

9:00 – 9:20 AM | Welcome Back & Day 1 Recap (20 min)

- Play 1-minute highlight reel from Day 1 (Rover dances, victory cheers).

- Quick circle: “What was your favorite moment yesterday? What are you most excited about today?”

- Teacher energy line: “Yesterday you made wheels move. Today we’re going to make legs walk — and tomorrow it gets even crazier with the Humanoid!”

9:20 – 10:00 AM | Quadruped Robotics Intro & Safety (40 min)

- Explain why legged robots are harder (balance, timing, more joints) but more capable.

- Live demo: Show Dog walking, trotting, turning — point out center of mass, leg sequencing.

- Safety talk:

  - Never put fingers near moving servos

  - Watch for servo overheating (feel carefully)

  - Power off before touching wiring

  - Keep workspace clear of loose cables

10:00 – 12:30 PM | Dog Mechanical & Electrical Assembly (2.5 hours – core hands-on block)

Step-by-step guided build (teacher demos on projector, students follow in teams):

- Frame & Leg Assembly (45 min)

  - Attach hip brackets, femur, tibia segments

  - Mount all 12 servos in correct orientation (use diagram)

  - Secure with screws — check for play

- Servo Wiring & Power (30 min)

  - Connect servos to PCA9685 PWM board (or direct Jetson if using fewer)

  - Wire power distribution (separate 5V logic, 7.4V+ servo power)

  - Safety check: no shorted wires

- Jetson Connection & Calibration (45 min)

  - Connect PWM board to Jetson I2C/GPIO

  - Run calibration script (zero all servos to neutral position)

  - Test one leg: move hip, knee, ankle individually

Basic Gait Coding (45 min)
Python
from adafruit_servokit import ServoKit

kit = ServoKit(channels=16)

# Example: set all legs to neutral

for i in range(12):

- kit.servo[i].angle = 90

  - Students code simple “stand up” sequence

12:30 – 1:30 PM | Lunch

1:30 – 3:30 PM | Gait Programming & Testing (2 hours)

- Teach basic trot/walk patterns (opposite legs move together)

- Students code & test:

  - Stand → sit sequence

  - Slow walk forward

  - Trot (faster alternating gait)

- Live tuning: adjust timing, angles, stride length

- Teacher tip: “When it falls, celebrate — that means you’re pushing limits!”

3:30 – 4:30 PM | Mission: “Formation Patrol” (60 min)

- Arena setup: 5-meter path + 30 cm wide bridge

- Objective: Rover leads, Dog follows at ~40 cm distance, both cross bridge without falling

- Success criteria:

  - Maintain formation for 5 meters

  - Bridge crossing without tipping

  - Record video of every successful run

- Teacher circulates, helps debug (common issues: gait timing, battery voltage drop)

4:30 – 5:00 PM | Reflection, Humanoid Teaser & Wrap-up (30 min)

- Group share: “What was hardest today? What felt amazing?”

- Quick Humanoid demo: walking, turning, simple arm gestures

- Homework: Review your code, think of improvements for tomorrow

- Hand out “Dog Builder” stickers/certificates

- Group photo with all Dogs “standing proud”

# Day 3

### Day 3 Learning Objectives

By the end of Day 3, students will be able to:

- Understand the difference between open-loop and closed-loop control.

- Wire and calibrate additional sensors (IMU, IR, limit switches) on Rover and Dog.

- Implement basic sensor reading, filtering, and data fusion.

- Code and tune a PID controller in Python on real Jetson hardware.

- Apply PID to real tasks (wall following, balance, speed control).

- See how the same PID concepts apply to Humanoid balance (demo).

### Materials Needed (per team of 4 students)

- Rover and Dog from Days 1–2 (fully functional)

- Additional sensors:

  - IMU (e.g., MPU6050 or BNO055)

  - IR distance/reflectance sensors (for line/edge detection)

  - Limit switches (mechanical bumpers)

- Jumper wires, breadboard or protoboard

- Screwdrivers, tape, hot glue gun (for sensor mounting)

- Jetson Orin Nano (from previous days)

- Monitor/keyboard or SSH laptop

- Printed PID tuning cheat sheet (simple table: increase Kp → faster, add Kd → less oscillation, etc.)

- Small obstacles, tape line on floor, small ramp (for testing)

Teacher Prep (night before):

- Test one Rover + Dog with IMU + IR working (wall follow + balance demo)

- Prepare 2-minute video: “Before PID vs. After PID” (Rover drifting → perfectly following line)

- Have spare sensors and cables

- Print simple wiring diagrams (IMU → I2C, IR → analog/digital pin)

### Detailed Day 3 Schedule (9:00 AM – 5:00 PM)

9:00 – 9:30 AM | Welcome Back & Day 2 Recap (30 min)

- Play 90-second highlight reel from Day 2 (Dogs walking, formation patrol cheers).

- Quick circle: “What was the hardest part of making the Dog walk? What felt awesome?”

- Teacher energy line: “Yesterday your Dog learned to walk. Today it learns to see and feel — and suddenly it becomes a smart robot!”

9:30 – 10:15 AM | Sensors & Feedback Intro (45 min)

- Explain open-loop vs. closed-loop with simple demo:

  - Open-loop: tell Rover “go forward 2 seconds” → crashes into wall

  - Closed-loop: Rover uses ultrasonic to stop before wall

- Show real-world examples: self-driving car sees lane → corrects steering

- Live demo: Rover with ultrasonic → stops before obstacle (wow moment)

- Introduce sensors:

  - Ultrasonic (distance)

  - IMU (orientation/acceleration)

  - IR (line/edge detection)

  - Limit switches (bumper)

10:15 – 10:30 AM | Break

10:30 AM – 12:30 PM | Sensor Wiring & Calibration (2 hours – core hands-on block)

Step-by-step guided build (teacher demos on projector):

- IMU Installation & Reading (30 min)

  - Wire IMU to Jetson I2C pins

Run test script:
Python
import board

import adafruit_mpu6050

i2c = board.I2C()

mpu = adafruit_mpu6050.MPU6050(i2c)

  - print(mpu.acceleration)

  - Students mount IMU on Rover & Dog

- IR & Limit Switch Wiring (30 min)

  - Connect IR sensors (analog/digital pins)

  - Add limit switches as bumpers

  - Test: print sensor values in terminal

- Sensor Fusion Basics (30 min)

  - Combine ultrasonic + IMU data

  - Simple filter: moving average to reduce noise

- Calibration & Safety Check (30 min)

  - Calibrate IMU zero point (flat surface)

  - Test all sensors live — students shout values as they move robots

12:30 – 1:30 PM | Lunch

1:30 – 3:30 PM | PID Control Deep Dive & Tuning (2 hours)

- Explain PID in simple terms:

  - P: Proportional – react to current error

  - I: Integral – fix past errors

  - D: Derivative – predict future errors

- Live demo: Rover wall following without PID (drifts) → with PID (smooth)

Students code basic PID loop:
Python
error = target_distance - current_distance

integral += error

derivative = error - last_error

output = Kp*error + Ki*integral + Kd*derivative

- robot.forward(output)

- Guided tuning:

  - Start with Kp only → watch oscillation

  - Add Kd → smooth it

  - Add Ki → eliminate steady-state error

- Apply to:

  - Rover wall following

  - Dog balance (keep upright)

3:30 – 4:30 PM | Mission: “Sensor Challenge Relay” (60 min)

- Arena setup: 4-meter path with moving obstacles, ramp, color block

- Objective:

  - Rover wall-follows & avoids dynamic obstacles

  - Hands off to Dog → climbs ramp using IMU balance

  - Humanoid detects red block & waves (teacher demo or student code)

- Success criteria:

  - Full relay under 3 minutes

  - No falls or collisions

  - Record video of every team’s run

4:30 – 5:00 PM | Reflection, Humanoid Teaser & Wrap-up (30 min)

- Group share: “What surprised you about PID? What felt like magic?”

- Quick Humanoid balance demo using same IMU/PID code

- Homework: Review your PID values, think of improvements

- Hand out “Sensor Master” stickers/certificates

- Group photo with all robots “reacting” to sensors

# Day 4

### Day 4 Learning Objectives

By the end of Day 4, students will be able to:

- Capture images from a camera attached to Jetson hardware.

- Use simple Python (Pillow/PIL) to check dominant color in an image region.

- Build a finite state machine (FSM) in Python to control robot behavior based on what it “sees”.

- Implement basic inter-robot communication (WiFi socket or Bluetooth).

- Integrate simple “vision” into decision-making.

- Run a coordinated multi-robot mission (Rover + Dog primary, Humanoid demo).

### Materials Needed (per team of 4 students)

- Rover and Dog from Days 1–3 (working)

- CSI camera module (Raspberry Pi Camera v2 or compatible for Jetson)

- USB WiFi dongle or Bluetooth module (for simple comms)

- Jetson Orin Nano (from previous days)

- Monitor/keyboard or SSH laptop

- Colored objects: large red/blue/green cards or balls (easy for average color check)

- Small arena with colored zones/obstacles

- Printed FSM template (simple boxes: Idle → Look → Approach → Signal)

Teacher Prep (night before):

- Test one Rover with camera + Pillow color check working.

- Pre-pair Bluetooth or prepare simple WiFi socket example.

- Prepare 2-minute video: “Simple Vision in Action” (robot sees red card → moves toward it).

- Have spare camera cables and large colored test objects.

### Detailed Day 4 Schedule (9:00 AM – 5:00 PM)

9:00 – 9:30 AM | Welcome Back & Day 3 Recap (30 min)

- Play 90-second highlight reel from Day 3 (PID magic, relay cheers).

- Quick circle: “When did your robot feel ‘smart’ yesterday? What are you most excited to try today?”

- Teacher energy line: “Yesterday your robots learned to sense and react. Today they get eyes — they’ll see color and decide what to do next!”

9:30 – 10:15 AM | Simple Vision Intro & Camera Setup (45 min)

- Explain: “Real robots need to see the world — but we’ll start super simple: just check if the middle of the image is mostly red.”

- Live demo: Show Jetson camera feed → hold red card → robot says “RED DETECTED!”

- Safety talk:

  - Camera lens is fragile — no touching

  - Avoid pointing at bright lights

- Guided setup:

  - Mount CSI camera on Rover (and optionally Dog)

Run basic camera test:
Python
from jetson_utils import videoSource, videoOutput

camera = videoSource("csi://0")

display = videoOutput("display://0")

while True:

img = camera.Capture()

if img is None:

break

  - display.Render(img)

  - Install Pillow if needed (already in JetPack usually): pip install pillow

10:15 – 10:30 AM | Break

10:30 AM – 12:30 PM | Simple Color Detection Hands-On (2 hours – core block)

Step-by-step guided coding (teacher demos on projector):

Capture & Show Image (20 min)
Python
from PIL import Image

import jetson_utils

camera = jetson_utils.videoSource("csi://0")

img = camera.Capture()

rgb_img = jetson_utils.cudaToNumpy(img)  # convert to numpy array

pil_img = Image.fromarray(rgb_img.astype('uint8'))

- pil_img.show()  # or save to file

- Check Average Color in Center Region (40 min)

  - Crop center 100×100 pixels

  - Calculate average RGB

  - Simple rule: if red channel > 150 and higher than green/blue → “RED DETECTED”

Python
center = pil_img.crop((width//2-50, height//2-50, width//2+50, height//2+50))

avg_color = center.convert('RGB').resize((1,1)).getpixel((0,0))

r, g, b = avg_color

if r > 150 and r > g and r > b:

- print("RED DETECTED!")

- Decision Integration (60 min)

  - If red detected → stop or turn toward it

  - Students test with different colors and distances

12:30 – 1:30 PM | Lunch

1:30 – 3:00 PM | Finite State Machines & Inter-Robot Comms (1.5 hours)

- Explain FSM: “Robots need clear states: Idle → Look for color → Approach → Signal teammate”

- Live demo: Rover FSM — Idle → See red → Approach → Send “FOUND” message

Students code basic FSM using the simple color check:
Python
state = "IDLE"

while True:

if state == "IDLE":

# capture image and check color

if red_detected:

state = "APPROACH"

elif state == "APPROACH":

robot.forward(0.3)

- # etc.

- Add simple comms (Bluetooth or socket):

  - Rover sends “FOUND” to Dog when red is detected

  - Dog reacts (follow or stop)

3:00 – 4:30 PM | Mission: “Multi-Robot Search” (90 min)

- Arena setup: scattered colored objects (large red/blue/green cards), rough patch, goal zone

- Objective:

  - Rover scouts & detects red card → signals Dog

  - Dog traverses rough patch to reach object

  - Humanoid (teacher demo) “retrieves” (or simulates pickup)

- Success criteria:

  - Red object located and “delivered” to start zone in under 4 minutes

  - Clean coordination (no collisions)

  - Record video of every team’s run

4:30 – 5:00 PM | Reflection, Humanoid Teaser & Wrap-up (30 min)

- Group share: “What surprised you about vision? How did FSM help you control behavior?”

- Quick Humanoid demo: gesture toward colored object using same simple color check

- Homework: Review your FSM code, think of one improvement

- Hand out “Vision Master” stickers/certificates

- Group photo with all robots “looking” at a colored object

### Teacher Tips for Maximum Engagement on Day 4

- Celebrate every “first red detection” — loud cheers when the message appears.

- Play upbeat music during coding time.

- Have a “color tuning station” for lighting adjustments (big colored cards help).

- Record mission runs — these are powerful portfolio clips.

- Emphasize: “This is how real robots see and decide — and you just built it!”

# Day 5

### Day 5 Learning Objectives

By the end of Day 5, students will be able to:

- Integrate all previous components (locomotion, sensing, simple vision, state machines, comms) into one cohesive system.

- Test and debug a complete multi-robot mission under time pressure.

- Collect meaningful performance data (run time, success rate, failure modes).

- Present their work professionally (problem → solution → demo → lessons).

- Understand real-world robotics ethics and responsibility.

- Feel ownership, pride, and excitement about continuing robotics.

### Materials Needed (per team of 4 students + full class)

- All Rovers, Dogs, and Humanoid from previous days (fully functional)

- Capstone arena: prepared Urban Search & Rescue setup

  - Flat zones (Rover scouting)

  - Rubble/stairs/ramps (Dog navigation)

  - “Victim” object (red/blue ball or tagged item)

  - Goal/safe zone

- Colored objects for simple vision check

- Stopwatches/timers

- Projector/screen for presentations

- Phone/camera tripods for recording

- “Level 1 Graduate” certificates + stickers

- Small prizes (stickers, ChatoorAI swag, candy) for best mission, best presentation, most creative solution

- Printed presentation template (Problem → Solution → Demo → Lessons → Future Ideas)

Teacher Prep (night before):

- Set up full arena and test one complete mission run (under 6 min)

- Prepare 3-minute “Level 1 Journey” montage video (clips from all 5 days)

- Have spare batteries, cables, screwdrivers ready

- Print certificates and judging rubric (fun categories: Fastest, Most Creative, Best Teamwork)

### Detailed Day 5 Schedule (9:00 AM – 5:00 PM)

9:00 – 9:45 AM | Welcome Back & Week Recap (45 min)

- Play 3-minute montage video (highlights from Days 1–4: Rover dance, Dog walking, PID magic, vision detection, relay cheers).

- Quick circle: “What was your favorite moment this week? What robot skill surprised you most?”

- Teacher energy line: “You started with unboxing on Monday. Today you become real roboticists — completing a full rescue mission with your own code and hardware!”

9:45 – 10:30 AM | Capstone Mission Briefing & Planning (45 min)

- Explain the full Urban Search & Rescue (USAR) scenario:

  - Rover scouts flat zones & detects object (simple color check)

  - Dog navigates rubble/stairs/ramps to reach object

  - Humanoid (teacher or advanced student demo) locates & “delivers” object to safe zone

- Success criteria (written on board):

  - Complete mission under 6 minutes

  - No critical failures (falls, collisions, lost comms)

  - Clean handoff between robots

  - Record video evidence

- Teams get 30 minutes to plan: assign roles, review code, make final tweaks

10:30 – 10:45 AM | Break

10:45 AM – 12:30 PM | Final Integration & Dry Runs (1 hour 45 min)

- Teams integrate all code (Rover vision → signal Dog → Dog navigation → Humanoid demo)

- Run 2–3 dry runs in arena (no time limit)

- Teacher circulates: help debug comms, timing, sensor issues

- Encourage creative solutions (e.g., Dog uses different gait for stairs)

12:30 – 1:30 PM | Lunch

1:30 – 3:30 PM | Live Capstone Missions & Recording (2 hours)

- Full class watches each team run (one at a time)

- Timer starts → mission begins

- Teacher narrates lightly: “Rover is scouting… Dog is approaching… Humanoid is delivering!”

- Record every run (phone on tripod)

- Immediate feedback: applause after each team finishes

- Allow 1–2 re-runs if time permits (best run counts)

3:30 – 4:30 PM | Presentations & Ethics Roundtable (60 min)

- Each team gets 8 minutes (strict timer):

  - Problem (1 min)

  - Solution & code highlights (2 min)

  - Live or recorded demo (3 min)

  - Lessons learned & future ideas (2 min)

- After all presentations: 15-minute ethics discussion (teacher leads):

  - “What if your robot made a wrong decision in a real rescue?”

  - “How do we make robots safe around people?”

  - “Who is responsible if a robot fails?”

  - Encourage thoughtful answers

4:30 – 5:00 PM | Celebration, Certificates & Closing (30 min)

- Award fun prizes:

  - Fastest Mission

  - Most Creative Solution

  - Best Teamwork

  - Best Presentation

- Hand out Level 1 Graduate certificates + “Dog Rescue Master” stickers

- Final group photo with all robots “celebrating” (e.g., all facing center)

- Closing words from teacher: “You didn’t just learn robotics — you became roboticists. Keep building. The future needs you.”

### Teacher Tips for Maximum Engagement on Day 5

- Keep the energy celebratory — this is their victory day.

- Play epic music during missions.

- Have a “mission control” table with timers and video setup.

- Record everything — these clips are portfolio gold and marketing material.

- End with genuine pride: “Look what you built in just one week!”

# Student Workbook Cover -HandoutsPage

### General Workbook Cover Page (Print as Page 1)

ChatoorAI Academy

Level 1: Jetson Foundations – Student Workbook

Name: ______________________________

Team Name: _________________________

Dates: [Start Date] – [End Date]

My Goals for Level 1:

- Build my first real robots

- Make them move, sense, and think

- Create a working rescue mission

- Have fun and learn like a real roboticist

My Why:

(Write 1–2 sentences: Why are you excited about robotics? What do you want to achieve?)

### Day 1 Handout – “Robot Unboxing Party + Rover Mastery”

Date: _______________

Today’s Goal: Make my Rover move and complete the first mission!

Key Things I Will Do Today:

- Watch live demos of all three robots

- Set up my Jetson Orin Nano

- Assemble and wire my Rover

- Write my first Python code

- Make my Rover do a victory dance

- Complete “Rover Recon” mission

My Team’s Robot Name: ________________________

My First Code Snippet (copy or write your version):

Python

# My Rover moves forward for 2 seconds

from jetbot import Robot

robot = Robot()

robot.forward(0.3)

time.sleep(2)

robot.stop()

Victory Dance Challenge – What did you make your Rover do?

(Draw or describe here)

Mission Tracker – Rover Recon

- Time to complete: ________ seconds

- Number of tries: ________

- Did I succeed? Yes / No

- What went wrong? ______________________________

- What I fixed: ______________________________

One thing I learned today:

One thing I’m excited for tomorrow:

Photo / Sketch: (space for student to glue photo or draw Rover)

### Day 2 Handout – “Dog Build & Locomotion – Main Project Day”

Date: _______________

Today’s Goal: Build my Dog robot and make it walk/trot!

Key Things I Will Do Today:

- Assemble the 12-servo Dog chassis

- Wire servos to Jetson

- Calibrate all joints

- Code walk and trot gaits

- Test formation with Rover

- Complete “Formation Patrol” mission

My Dog’s Name: ________________________

My Favorite Gait Code Snippet (copy or write):

Python

# Example trot gait (simplified)

kit.servo[0].angle = 90   # hip

kit.servo[1].angle = 100  # knee

time.sleep(0.2)

# ... continue for all legs

Mission Tracker – Formation Patrol

- Distance traveled in formation: ________ meters

- Did we cross the bridge? Yes / No

- Time to complete: ________ seconds

- What was hardest? ______________________________

- What worked well? ______________________________

Reflection:

- How is walking different from rolling?

- One thing I learned about servos:

Photo / Sketch: (space for Dog photo or drawing)

### Day 3 Handout – “Sensors, PID & Closed-Loop Control”

Date: _______________

Today’s Goal: Make my robots react to the world with sensors and PID!

Key Things I Will Do Today:

- Add IMU, IR, and limit switches

- Read and filter sensor data

- Code and tune PID controller

- Test wall following & balance

- Complete “Sensor Challenge Relay”

My PID Values (write what you tuned):

Kp = ____   Ki = ____   Kd = ____

Mission Tracker – Sensor Challenge Relay

- Total time: ________ minutes

- Number of handoffs: ________

- Did everyone finish? Yes / No

- Biggest challenge: ______________________________

- How I fixed it: ______________________________

Reflection:

- What does “closed-loop” mean in simple words?

- When did your robot feel “smart” today?

Photo / Sketch: (space for sensor setup or relay moment)

### Day 4 Handout – “Vision, State Machines & Integration”

Date: _______________

Today’s Goal: Make my robots see color and decide what to do!

Key Things I Will Do Today:

- Mount camera on Rover/Dog

- Use simple Python to detect color

- Build a finite state machine (FSM)

- Add basic inter-robot communication

- Complete “Multi-Robot Search” mission

My Simple Vision Code Snippet (copy or write):

Python

# Check if center is red

avg_color = center.getpixel((0,0))  # from PIL

r, g, b = avg_color

if r > 150 and r > g and r > b:

print("RED DETECTED!")

Mission Tracker – Multi-Robot Search

- Time to find and deliver object: ________ minutes

- Did we succeed? Yes / No

- Coolest moment: ______________________________

- One improvement idea: ______________________________

Reflection:

- How did FSM help control my robot?

- What surprised you about “vision”?

Photo / Sketch: (space for robot “seeing” color)

### Day 5 Handout – “Capstone Integration, Testing & Showcase”

Date: _______________

Today’s Goal: Complete the full rescue mission and show the world what I built!

Key Things I Will Do Today:

- Integrate all code from the week

- Test & debug the full mission

- Run the live Urban Search & Rescue

- Present my team’s work

- Celebrate and reflect

My Team’s Final Mission Time: ________ minutes

Our Best Run: Yes / No (circle one)

My Presentation Notes:

- Problem we solved: ______________________________

- How we did it: ______________________________

- What I learned this week: ______________________________

- Future idea: ______________________________

Reflection – My Level 1 Journey

- Favorite moment: ______________________________

- Hardest moment: ______________________________

- Biggest thing I learned: ______________________________

- How I feel about robotics now: ______________________________

Photo / Sketch: (space for capstone mission photo or team picture)

Certificate Reminder:

Don’t forget to collect your official Level 1 Graduate certificate today!

### How to Use These Handouts

Print Options:

- Daily packets — 1–2 pages per day (print 5 separate sheets)

- Full workbook — Combine into one 10–12 page booklet (cover + 5 days + reflection)

In Classroom:

- Give out at the start of each day

- Students fill in during reflection (last 15 min)

- Collect at end of day (optional) for feedback/portfolio building

- Encourage photos/gluing mission pics in the spaces

Digital Version:

- Share as Google Doc template — students type directly

- Or PDF fillable fields for name, mission times, notes

# Teacher Workbook – Level 1: Jetson Foundations

### Teacher Workbook – Level 1: Jetson Foundations

For Instructor Use Only

Cover Page (same as student version – no changes needed)

### Day 1 Handout – Teacher Notes

Student Content (same as student version – shown here for reference):

Today’s Goal: Make my Rover move and complete the first mission!

Key Things I Will Do Today:

- Watch live demos of all three robots

- Set up my Jetson Orin Nano

- Assemble and wire my Rover

- Write my first Python code

- Make my Rover do a victory dance

- Complete “Rover Recon” mission

My Team’s Robot Name: ________________________

My First Code Snippet:

Python

# My Rover moves forward for 2 seconds

from jetbot import Robot

robot = Robot()

robot.forward(0.3)

time.sleep(2)

robot.stop()

Victory Dance Challenge – What did you make your Rover do?

(Draw or describe here)

Mission Tracker – Rover Recon

- Time to complete: ________ seconds

- Number of tries: ________

- Did I succeed? Yes / No

- What went wrong? ______________________________

- What I fixed: ______________________________

One thing I learned today:

One thing I’m excited for tomorrow:

Photo / Sketch: (space)

Teacher Notes & Answer Key for Day 1

Overall goal: Create instant “wow” and ownership. Celebrate every small win loudly — this builds momentum for the week.

Common student questions / troubleshooting:

- “Why is my Jetson not booting?” → Check power supply (must be 5V 4A+), SSD connection, or re-flash if needed.

- “My motors don’t move!” → Double-check wiring polarity, GPIO pin numbers, battery voltage.

- “Code error: Robot not found” → Ensure jetbot library is installed (pip install jetbot if missing).

Expected code variations (accept any that work):

- Students may change speed (0.3 → 0.5) or time (2 → 3)

- Victory dance ideas: spin left/right, zigzag, stop-start, reverse

Mission success criteria (for judging):

- Completes course without hitting obstacles

- Stops within ±10 cm of target (be lenient first day)

- At least one clean run recorded

Reflection prompts to ask verbally:

- “Raise your hand if you felt proud when your Rover moved!”

- “Who had to debug something? What did you learn?”

Differentiation:

- Struggling students: Pair with stronger coder for scripting part

- Advanced students: Challenge to add reverse or turn in victory dance

### Day 2 Handout – Teacher Notes

Student Content (reference):

Today’s Goal: Build my Dog robot and make it walk/trot!

Key Things I Will Do Today:

- Assemble the 12-servo Dog chassis

- Wire servos to Jetson

- Calibrate all joints

- Code walk and trot gaits

- Test formation with Rover

- Complete “Formation Patrol” mission

My Dog’s Name: ________________________

My Favorite Gait Code Snippet:

Python

# Example trot gait (simplified)

kit.servo[0].angle = 90   # hip

kit.servo[1].angle = 100  # knee

time.sleep(0.2)

# ... continue for all legs

Mission Tracker – Formation Patrol

- Distance traveled in formation: ________ meters

- Did we cross the bridge? Yes / No

- Time to complete: ________ seconds

- What was hardest? ______________________________

- What worked well? ______________________________

Reflection:

- How is walking different from rolling?

- One thing I learned about servos:

Photo / Sketch:

Teacher Notes & Answer Key for Day 2

Overall goal: Students feel the thrill of legged movement. Expect lots of falls — turn them into learning moments.

Common issues / fixes:

- “Servos jitter” → Check power supply (needs 7.4V+ stable), add capacitors if needed

- “Dog falls forward” → Recalibrate neutral angles (usually 90°, but varies)

- “Code runs but no movement” → Verify PCA9685 address (default 0x40) and channel mapping

Expected gait code:

- No single “correct” version — accept any that produces forward motion

Good example (simplified trot):
Python
def trot_step():

# Front left & rear right up

kit.servo[0].angle = 110

kit.servo[6].angle = 110

time.sleep(0.2)

- # etc.

Mission success criteria:

- Maintain ~30–50 cm distance for at least 3 meters

- Bridge crossing without fall

- Encourage creativity (e.g., Dog speed variation)

Reflection prompts:

- “Who had a fall? What did you learn from it?”

- “How did it feel when your Dog first walked?”

Differentiation:

- Struggling teams: Provide pre-calibrated servo angles

- Advanced: Challenge to add reverse gait or turn

### Day 3 Handout – Teacher Notes

Student Content (reference):

Today’s Goal: Make my robots react to the world with sensors and PID!

Key Things I Will Do Today:

- Add IMU, IR, and limit switches

- Read and filter sensor data

- Code and tune PID controller

- Test wall following & balance

- Complete “Sensor Challenge Relay”

My PID Values:

Kp = ____   Ki = ____   Kd = ____

Mission Tracker – Sensor Challenge Relay

- Total time: ________ minutes

- Number of handoffs: ________

- Did everyone finish? Yes / No

- Biggest challenge: ______________________________

- How I fixed it: ______________________________

Reflection:

- What does “closed-loop” mean in simple words?

- When did your robot feel “smart” today?

Photo / Sketch:

Teacher Notes & Answer Key for Day 3

Overall goal: Students experience the “aha” of feedback control.

Common PID tuning results:

- Too high Kp → oscillation (wobbling)

- Too high Ki → overshoot or windup

- Kd helps dampen oscillation

- Typical stable values (Rover wall follow): Kp=0.8–1.5, Ki=0.01–0.05, Kd=0.2–0.6

Mission success criteria:

- Full relay completed

- Rover avoids obstacles

- Dog climbs ramp without tipping

- Humanoid wave detected (even if teacher-assisted)

Reflection answers (guide verbally):

- Closed-loop: “Robot uses sensors to correct itself instead of just following blind commands.”

- Smart moment: “When it followed the wall perfectly after tuning.”

Differentiation:

- Struggling: Use pre-tuned PID values

- Advanced: Add feedforward term or try different sensor fusion

### Day 4 Handout – Teacher Notes

Student Content (reference):

Today’s Goal: Make my robots see color and decide what to do!

Key Things I Will Do Today:

- Mount camera on Rover/Dog

- Use simple Python to detect color

- Build a finite state machine (FSM)

- Add basic inter-robot communication

- Complete “Multi-Robot Search” mission

My Simple Vision Code Snippet:

Python

# Check if center is red

avg_color = center.getpixel((0,0))

r, g, b = avg_color

if r > 150 and r > g and r > b:

print("RED DETECTED!")

Mission Tracker – Multi-Robot Search

- Time to find and deliver object: ________ minutes

- Did we succeed? Yes / No

- Coolest moment: ______________________________

- One improvement idea: ______________________________

Reflection:

- How did FSM help control my robot?

- What surprised you about “vision”?

Photo / Sketch:

Teacher Notes & Answer Key for Day 4

Overall goal: Students see robots “think” — simple but powerful.

Common issues / fixes:

- “Color detection not working” → Check lighting (use consistent room light), adjust threshold (r > 140 instead of 150)

- “FSM stuck” → Add print(state) to debug current state

- “Comms not sending” → Verify IP addresses or Bluetooth pairing

Expected vision code variations:

- Students may adjust crop size or threshold — accept any that reliably detects red

Mission success criteria:

- Rover detects color → signals Dog

- Dog reaches object area

- Object “delivered” (even simulated)

Reflection answers:

- FSM helps: “It organizes what the robot does next instead of random code.”

- Vision surprise: “It’s harder than I thought — lighting changes everything!”

Differentiation:

- Struggling: Provide pre-written color check function

- Advanced: Add multiple colors or size-based distance estimation

### Day 5 Handout – Teacher Notes

Student Content (reference):

Today’s Goal: Complete the full rescue mission and show the world what I built!

Key Things I Will Do Today:

- Integrate all code from the week

- Test & debug the full mission

- Run the live Urban Search & Rescue

- Present my team’s work

- Celebrate and reflect

My Team’s Final Mission Time: ________ minutes

Our Best Run: Yes / No (circle one)

My Presentation Notes:

- Problem we solved: ______________________________

- How we did it: ______________________________

- What I learned this week: ______________________________

- Future idea: ______________________________

Reflection – My Level 1 Journey

- Favorite moment: ______________________________

- Hardest moment: ______________________________

- Biggest thing I learned: ______________________________

- How I feel about robotics now: ______________________________

Photo / Sketch:

Teacher Notes & Answer Key for Day 5

Overall goal: Celebration + reflection. End on a high note.

Mission judging rubric (use this to award prizes):

- Completion time (fastest = prize)

- Creativity (unique solution = prize)

- Teamwork (best collaboration = prize)

- Presentation clarity (best speaker = prize)

Common presentation strengths to praise:

- Clear explanation of problem/solution

- Honest lessons learned

- Fun demo (even if imperfect)

Reflection prompts (facilitate verbally):

- “Who feels more confident in robotics now than on Day 1?”

- “What skill do you want to keep practicing at home?”

Closing note to students:

“Five days ago you unboxed robots. Today you built intelligent systems. That’s incredible growth. Keep building — the world needs roboticists like you.”

This teacher version is now complete for all 5 days — fully ready to guide your instructors.

# Mandatory Reading & Agreement – All Participants

### ChatoorAI Academy

Level 1: Jetson Foundations – Safety Instructions & Rules

Mandatory Reading & Agreement – All Participants

Workshop Dates: ___________________________

Student Name: ______________________________

Parent/Guardian Name (if under 18): ______________________________

Signature (student & parent/guardian): ______________________________ Date: ________

1. General Safety Principles

- Respect the equipment. All robots, Jetson boards, servos, batteries, and tools are real commercial-grade hardware — not toys. Treat them with care.

- Never work alone on powered hardware. Always have a teammate or instructor nearby when power is on.

- No running, pushing, or horseplay in the lab area — serious injury or damage can occur.

- Keep long hair tied back, remove loose jewelry, and wear closed-toe shoes.

2. Electrical & Battery Safety

- Never touch live wires or connectors while power is on.

- LiPo batteries are dangerous if mishandled:

  - Do not puncture, bend, crush, short-circuit, or overcharge.

  - Immediately report any swelling, heat, smoke, or smell — stop using and alert instructor.

  - Only charge batteries with the approved balance charger under instructor supervision.

  - Never leave charging batteries unattended.

- Power off everything before connecting/disconnecting cables or changing wiring.

- Use only provided power supplies — no phone chargers or random cables.

3. Moving Parts & Servo Safety

- Never put fingers, hair, clothing, or objects near moving servos, motors, or joints while powered.

- Servos can get hot — do not touch after prolonged use; allow to cool.

- Power off immediately if a servo is stalling, buzzing loudly, or overheating.

- Keep robot workspace clear — no loose cables, tools, or body parts in the path of motion.

4. Jetson & Computer Safety

- Do not force USB/HDMI cables — pins are delicate.

- No food, drinks, or liquids anywhere near Jetson boards or electronics.

- Do not cover vents — Jetson can overheat if airflow is blocked.

- Log out / power down properly when finished — never yank power cord.

5. Eye & Personal Safety

- Wear safety glasses when assembling (screws, servo gears, sharp edges).

- Report any cut, burn, shock, or injury immediately — even if minor.

- First-aid kit is located at [specify location] — instructor has access.

6. Respect & Lab Rules

- Follow instructor directions at all times — especially during powered tests.

- No unauthorized modifications — do not remove components or change wiring without permission.

- Clean your workspace at the end of each day — return tools, sweep screws, organize cables.

- No photos/videos of other students without their explicit permission.

Consequences for Safety Violations

1st violation: Verbal warning + re-read rules with instructor

2nd violation: Loss of hands-on time (observation only) + parent notification

3rd violation: Removal from workshop (no refund) + parent/guardian meeting

Acknowledgment

I have read, understood, and agree to follow these safety rules. I understand that failure to do so may result in removal from the workshop for safety reasons.

Student Signature: ______________________________ Date: ________

Parent/Guardian Signature (if under 18): ______________________________ Date: ________

Instructor Signature (upon review): ______________________________ Date: ________

### How to Use & Enforce This Document

- Day 1 Morning (first 15 minutes):

  - Every student reads silently (or aloud in small groups)

  - Instructor reads key points dramatically and shows examples (e.g., hot servo, battery safety demo)

  - Students sign + parent signs if applicable

- Post visibly

  - Print large version (A3 or poster size) and place at entrance + each table

  - Laminate for reuse

- Daily reminder

  - Quick 1-minute recap each morning: “What are the three most important safety rules today?”

- Enforcement

  - Be consistent and fair — safety is non-negotiable

  - Document any violation (date, student name, rule broken, action taken)

# Parent / Guardian Information & Consent Form

ChatoorAI Academy – Level 1: Jetson Foundations

Parent / Guardian Information & Consent Form

Workshop Title: Level 1: Jetson Foundations – Real NVIDIA Jetson Robotics

Dates: [Insert Start & End Date, e.g., March 10–14, 2026]

Location: [Insert Venue / Lab Address]

Daily Schedule: 9:00 AM – 5:00 PM (with breaks & lunch provided)

Age Group: 15+ (high-school & early-college students)

Dear Parent / Guardian,

Thank you for considering ChatoorAI Academy for your teen. This 5-day intensive workshop gives participants hands-on experience building and programming real commercial-grade robots using NVIDIA Jetson hardware — the same platform used in autonomous vehicles, service robots, and research labs worldwide.

Program Highlights

- Students physically assemble, wire, and debug three robot types: wheeled Rover, quadruped Dog, and bipedal Humanoid (Humanoid is demonstrated daily).

- 80–90% of each day is dedicated to hands-on work with real hardware — no toy kits.

- Focus: Python coding, sensors, basic control loops, simple vision, teamwork, and a capstone rescue mission.

- Outcome: Portfolio videos, code ownership, official Level 1 certificate, and 3 months of home simulation access.

Safety & Supervision

Safety is our highest priority. Every participant must follow strict safety rules (see attached or Day 1 handout). Key points include:

- No touching moving parts while powered

- Careful handling of LiPo batteries (no puncturing, shorting, or leaving unattended while charging)

- Power off before any wiring changes

- Safety glasses required during assembly

- Immediate reporting of any injury or equipment issue

All sessions are led by experienced instructors with background checks. There is a 1:6 instructor-to-student ratio (or better). First-aid kits and emergency procedures are in place.

Liability & Consent

I, the undersigned parent/guardian, give permission for my child named below to participate in the ChatoorAI Academy Level 1 workshop. I understand:

- The activities involve real electronic hardware, tools, moving parts, and batteries, which carry inherent risks (minor cuts, burns, electrical shock, etc.) despite all safety measures.

- My child will be supervised at all times, but I accept responsibility for any risks associated with participation.

- ChatoorAI Academy, its staff, and venue are not liable for injury, loss, or damage unless caused by gross negligence.

- Photos/videos may be taken during the workshop for educational/portfolio purposes and may be used (anonymously or with permission) for marketing. I can opt out by checking below.

Student Information

Full Name: ______________________________

Age: _____  Date of Birth: ________________

Emergency Contact Name: ______________________________

Emergency Phone: ______________________________

Any medical conditions / allergies we should know about?

Parent / Guardian Consent

I have read and understand the program description and safety rules. I give permission for my child to participate and use all equipment under supervision.

Signature: ______________________________ Date: ________

Printed Name: ______________________________

Photo / Video Release (optional)

☐ Yes – I grant permission for photos/videos of my child to be taken and used (anonymously or named) for ChatoorAI marketing, portfolio, or social media.

☐ No – Do not use my child’s image or name in any promotional materials.

Payment & Cancellation

Workshop fee: $[Insert Amount] per participant

Payment due by: [Date]

Cancellation policy: Full refund if canceled 14+ days prior; 50% refund 7–13 days prior; no refund within 7 days.

Contact Information

Questions? Email: info@chatoorai.academy

Phone: [Insert Number]

Website: www.chatoorai.academy

Thank you for trusting ChatoorAI Academy with your teen’s learning journey. We are committed to a safe, exciting, and transformative experience.

Instructor Acknowledgment (to be signed on Day 1)

I have reviewed this form with the participant and parent/guardian.

Signature: ______________________________ Date: ________